VLSI DESIGN AND AUTOMATION SUMMER INTERNSHIP

WEEK 4 SUBMISSION

RESOURCE: WEEK 1(ENC,NETLIST and DATA files are here)

VERILOG CODE AND WAVE FORM

- AND Gate

module and_gate(

input a, b,

output reg out);

always @(*) begin

if (a==1'b1 && b==1'b1)

out = 1'b1;

else

out = 1'b0;

end

endmodule

TB

module gate_and_TB();

reg a,b;

wire out;

and_gate dut(a, b, out);

initial begin

a = 0;

b = 0;

#10;

a = 0;

b = 1;

#10;

a = 1;

b = 0;

#10;

a = 1;

b = 1;

#10;

//$finish;

end

endmodule

- OR gate

module gate_or(

input a, b,

output reg out);

always @(*) begin

if (a==1'b0 && b==1'b0)

out = 1'b0;

else

out = 1'b1;

end

endmodule

TB

module gate_or_tb;

reg a, b;

wire out;

gate_or uut (

.a(a),

.b(b),

.out(out)

);

initial begin

a = 0; b = 0; #10;

a = 0; b = 1; #10;

a = 1; b = 0; #10;

a = 1; b = 1; #10;

$finish;

end

endmodule

- NOT gate

module gate_not(

input in,

output out);

assign out = ~in;

endmodule

TB

module gate_not_tb;

reg in;

wire out;

gate_not uut (

.in(in),

.out(out)

);

initial begin

in = 0; #10;

in = 1; #10;

$finish;

end

endmodule

- NAND Gate

module nand_gate(

input a, b,

output reg out);

always @(*) begin

if (a==1'b1 && b==1'b1)

out = 1'b0;

else

out = 1'b1;

end

endmodule

TB

module nand_gate_tb;

reg a, b;

wire out;

nand_gate uut (

.a(a),

.b(b),

.out(out)

);

initial begin

a = 0; b = 0; #10;

a = 0; b = 1; #10;

a = 1; b = 0; #10;

a = 1; b = 1; #10;

$finish;

end

endmodule

- NOR Gate

module gate_nor(

input a, b,

output reg out);

always @(*) begin

if (a==1'b0 && b==1'b0)

out = 1'b1;

else

out = 1'b0;

end

endmodule

TB

module gate_nor_tb;

reg a, b;

wire out;

gate_nor uut (

.a(a),

.b(b),

.out(out)

);

initial begin

a = 0; b = 0; #10;

a = 0; b = 1; #10;

a = 1; b = 0; #10;

a = 1; b = 1; #10;

$finish;

end

endmodule

- XOR Gate

module gate_xor(

input a, b,

output reg out);

always @(*) begin

if ((a==1'b0 && b==1'b0) || (a==1'b1 && b==1'b1))

out = 1'b0;

else

out = 1'b1;

end

endmodule

TB

module gate_xor_tb;

reg a, b;

wire out;

gate_xor uut (

.a(a),

.b(b),

.out(out)

);

initial begin

a = 0; b = 0; #10;

a = 0; b = 1; #10;

a = 1; b = 0; #10;

a = 1; b = 1; #10;

$finish;

end

endmodule

- XNOR Gate

module gate_xnor(

input a, b,

output reg out);

always @(*) begin

if ((a==1'b0 && b==1'b0) || (a==1'b1 && b==1'b1))

out = 1'b1;

else

out = 1'b0;

end

endmodule

TB

module gate_xnor_tb;

reg a, b;

wire out;

gate_xnor uut (

.a(a),

.b(b),

.out(out)

);

initial begin

a = 0; b = 0; #10;

a = 0; b = 1; #10;

a = 1; b = 0; #10;

a = 1; b = 1; #10;

$finish;

end

endmodule

- Gate Combo

module gate_combo(

input a, b,

output out);

wire w0, w1, w2;

assign w0 = a^b;

assign w1 = w0&a;

assign w2 = w1|w0;

assign out = ~w2;

endmodule

TB

module gate_combo_tb;

reg a, b;

wire out;

gate_combo uut (

.a(a),

.b(b),

.out(out)

);

initial begin

a = 0; b = 0; #10;

a = 0; b = 1; #10;

a = 1; b = 0; #10;

a = 1; b = 1; #10;

$finish;

end

endmodule

- 4 input AND Gate

module and4_gate(

input a, b, c, d,

output reg out);

always @(*) begin

if (a==1'b1 && b==1'b1 && c==1'b1 && d==1'b1)

out = 1'b1;

else

out = 1'b0;

end

endmodule

TB

module and4_gate_tb;

reg a, b, c, d;

wire out;

and4_gate uut (

.a(a),

.b(b),

.c(c),

.d(d),

.out(out)

);

initial begin

a = 0; b = 0; c = 0; d = 0; #10;

a = 0; b = 1; c = 1; d = 1; #10;

a = 1; b = 0; c = 1; d = 1; #10;

a = 1; b = 1; c = 0; d = 1; #10;

a = 1; b = 1; c = 1; d = 0; #10;

a = 1; b = 1; c = 1; d = 1; #10;

$finish;

end

endmodule

- 4 Input OR Gate

module gate_4or(

input a, b, c, d,

output reg out);

always @(*) begin

if (a==1'b0 && b==1'b0 && c==1'b0 && d==1'b0)

out = 1'b0;

else

out = 1'b1;

end

endmodule

TB

module gate_4or_tb;

reg a, b, c, d;

wire out;

gate_4or uut (

.a(a),

.b(b),

.c(c),

.d(d),

.out(out)

);

initial begin

a = 0; b = 0; c = 0; d = 0; #10;

a = 1; b = 0; c = 0; d = 0; #10;

a = 0; b = 1; c = 0; d = 0; #10;

a = 0; b = 0; c = 1; d = 0; #10;

a = 0; b = 0; c = 0; d = 1; #10;

a = 1; b = 1; c = 1; d = 1; #10;

$finish;

end

endmodule

- Half Adder

module ha(

input a, b,

output sum, carry);

assign sum = a^b;

assign carry = a&b;

endmodule

TB

module ha_tb;

reg a, b;

wire sum, carry;

ha uut (

.a(a),

.b(b),

.sum(sum),

.carry(carry)

);

initial begin

a = 0; b = 0; #10;

a = 0; b = 1; #10;

a = 1; b = 0; #10;

a = 1; b = 1; #10;

$finish;

end

endmodule

- Full Adder

module fa(

input a, b, c,

output sum, carry);

assign sum = a^b^c;

assign carry = (a&b) | (b&c) | (c&a);

endmodule

TB

module fa_tb;

reg a, b, c;

wire sum, carry;

fa uut (

.a(a),

.b(b),

.c(c),

.sum(sum),

.carry(carry)

);

initial begin

a = 0; b = 0; c = 0; #10;

a = 0; b = 0; c = 1; #10;

a = 0; b = 1; c = 0; #10;

a = 0; b = 1; c = 1; #10;

a = 1; b = 0; c = 0; #10;

a = 1; b = 0; c = 1; #10;

a = 1; b = 1; c = 0; #10;

a = 1; b = 1; c = 1; #10;

$finish;

end

endmodule

- 4 Bit Ripple Carry Adder

module rca4(

input [3:0] a, b,

input cin,

output [3:0] sum,

output cout);

wire [2:0] w;

fa u0(a[0], b[0], cin, sum[0], w[0]);

fa u1(a[1], b[1], w[0], sum[1], w[1]);

fa u2(a[2], b[2], w[1], sum[2], w[2]);

fa u3(a[3], b[3], w[2], sum[3], cout);

endmodule

TB

module rca4_tb;

reg [3:0] a, b;

reg cin;

wire [3:0] sum;

wire cout;

rca4 uut (

.a(a),

.b(b),

.cin(cin),

.sum(sum),

.cout(cout)

);

initial begin

a = 4'b0000; b = 4'b0000; cin = 0; #10;

a = 4'b0001; b = 4'b0010; cin = 0; #10;

a = 4'b0101; b = 4'b0011; cin = 1; #10;

a = 4'b1111; b = 4'b0001; cin = 0; #10;

a = 4'b1010; b = 4'b0101; cin = 1; #10;

$finish;

end

endmodule

- 4 Bit Subtractor

module sub4(

input [3:0] a, b,

output [3:0] diff,

output borrow

);

wire [4:0] temp;

assign temp = {1'b0, a} - {1'b0, b};

assign diff = temp[3:0];

assign borrow = temp[4];

endmodule

TB

module sub4TB;

reg [3:0] a, b;

wire [3:0] diff;

wire borrow;

sub4 uut (

.a(a),

.b(b),

.diff(diff),

.borrow(borrow)

);

initial begin

a = 4'b0110; b = 4'b0011; #10;

a = 4'b1000; b = 4'b0100; #10;

a = 4'b0011; b = 4'b1000; #10;

a = 4'b1111; b = 4'b0001; #10;

$finish;

end

endmodule

- 4 Bit ADDER Subtractor

module add_sub4(

input [3:0] a, b,

input sel,             // sel = 0 for addition, sel = 1 for subtraction

output [3:0] result,

output carry_borrow

);

wire [3:0] b_xor;

wire cin;

assign b_xor = b ^ {4{sel}};  // XOR b with sel: if sel=1, invert b (2's comp)

assign cin = sel;

assign {carry_borrow, result} = a + b_xor + cin;

endmodule

TB

module add_sub4TB;

reg [3:0] a, b;

reg sel;

wire [3:0] result;

wire carry_borrow;

add_sub4 uut (

.a(a),

.b(b),

.sel(sel),

.result(result),

.carry_borrow(carry_borrow)

);

initial begin

a = 4'b0101; b = 4'b0011; sel = 0; #10;  // 5 + 3

a = 4'b1000; b = 4'b0100; sel = 0; #10;  // 8 + 4

a = 4'b0110; b = 4'b0011; sel = 1; #10;  // 6 - 3

a = 4'b0010; b = 4'b0101; sel = 1; #10;  // 2 - 5

$finish;

end

endmodule

- Mux 2X1

module mux_2X1(

input a, b,sel,

output q

);

assign q = (sel) ? b : a;

endmodule

TB

module mux_2X1_TB();

reg a, b, sel;

wire q;

mux_2X1 dut(a, b, sel ,q);

initial begin

a = 0;

b = 0;

sel = 0;

#10;

a = 1;

b = 0;

sel = 0;

#10;

a = 1;

b = 0;

sel = 1;

#10;

a = 0;

b = 1;

sel = 0;

#10;

a = 0;

b = 1;

sel = 1;

#10;

end

endmodule

- MUX 4X1

module mux_4X1(

input [3:0] in,

input [1:0] sel,

output reg q);

always @(*) begin

case (sel)

2'b00 : q = in[0];

2'b01 : q = in[1];

2'b10 : q = in[2];

2'b11 : q = in[3];

endcase

end

endmodule

TB

module mux_4X1_tb;

reg [3:0] in;

reg [1:0] sel;

wire q;

mux_4X1 uut (

.in(in),

.sel(sel),

.q(q)

);

initial begin

in = 4'b1010;

sel = 2'b00; #10;

sel = 2'b01; #10;

sel = 2'b10; #10;

sel = 2'b11; #10;

$finish;

end

endmodule

- MUX 8X1

module mux_8X1(

input [7:0] in,

input [2:0] sel,

output reg q);

always @(*) begin

case (sel)

3'b000 : q = in[0];

3'b001 : q = in[1];

3'b010 : q = in[2];

3'b011 : q = in[3];

3'b100 : q = in[4];

3'b101 : q = in[5];

3'b110 : q = in[6];

3'b111 : q = in[7];

endcase

end

endmodule

TB

- DEMUX 1X2

module demux_1X2(

input a, sel,

output [1:0] out);

assign out = a << sel;

endmodule

TB

module demux_1X2_tb;

reg a, sel;

wire [1:0] out;

demux_1X2 uut (

.a(a),

.sel(sel),

.out(out)

);

initial begin

a = 0; sel = 0; #10;

a = 1; sel = 0; #10;

a = 1; sel = 1; #10;

a = 0; sel = 1; #10;

$finish;

end

endmodule

- DEMUX 1X4

module demux_1X4(

input a,

input [1:0] sel,

output reg [3:0] q);

//assign q = a << sel;

always @(*) begin

case (sel)

2'b00 : q[0] = a;

2'b01 : q[1] = a;

2'b10 : q[2] = a;

2'b11 : q[3] = a;

endcase

end

endmodule

TB

module demux_1X4_tb;

reg a;

reg [1:0] sel;

wire [3:0] q;

demux_1X4 uut (

.a(a),

.sel(sel),

.q(q)

);

initial begin

a = 1;

sel = 2'b00; #10;

sel = 2'b01; #10;

sel = 2'b10; #10;

sel = 2'b11; #10;

a = 0; sel = 2'b10; #10;

$finish;

end

endmodule

- Decoder 2X4

module decod_2X4(

input [1:0] a,

output reg [3:0] out);

always @(*) begin

case (a)

2'b00 : out[0] = 1'b1;

2'b01 : out[1] = 1'b1;

2'b10 : out[2] = 1'b1;

2'b11 : out[3] = 1'b1;

endcase

end

endmodule

TB

module decod_2X4_tb;

reg [1:0] a;

wire [3:0] out;

decod_2X4 uut (

.a(a),

.out(out)

);

initial begin

a = 2'b00; #10;

a = 2'b01; #10;

a = 2'b10; #10;

a = 2'b11; #10;

$finish;

end

endmodule

- Decoder 3X8

module decod_3X8(

input [2:0] a,

output  [7:0] out);

assign out = 8'b00000001 << a;

endmodule

TB

module decod_3X8_tb;

reg [2:0] a;

wire [7:0] out;

decod_3X8 uut (

.a(a),

.out(out)

);

initial begin

a = 3'b000; #10;

a = 3'b001; #10;

a = 3'b010; #10;

a = 3'b011; #10;

a = 3'b100; #10;

a = 3'b101; #10;

a = 3'b110; #10;

a = 3'b111; #10;

$finish;

end

endmodule

- Encoder 4X2

module encod_4X2(

input [3:0] a,

output reg [1:0] out);

always @(*) begin

case (a)

4'b0001 : out = 2'b00;

4'b0010 : out = 2'b01;

4'b0100 : out = 2'b10;

4'b1000 : out = 2'b11;

default : out = 2'b00;

endcase

end

endmodule

TB

module encod_4X2_tb;

reg [3:0] a;

wire [1:0] out;

encod_4X2 uut (

.a(a),

.out(out)

);

initial begin

a = 4'b0001; #10;

a = 4'b0010; #10;

a = 4'b0100; #10;

a = 4'b1000; #10;

a = 4'b0000; #10;

a = 4'b1111; #10;

$finish;

end

endmodule

- Encoder 8X3

module encod_8X3(

input [7:0] a,

output reg [2:0] out);

always @(*) begin

case (a)

8'b00000001 : out = 3'b000;

8'b00000010 : out = 3'b001;

8'b00000100 : out = 3'b010;

8'b00001000 : out = 3'b011;

8'b00010000 : out = 3'b100;

8'b00100000 : out = 3'b101;

8'b01000000 : out = 3'b110;

8'b10000000 : out = 3'b111;

default : out = 3'b000;

endcase

end

endmodule

TB

module encod_8X3_tb;

reg [7:0] a;

wire [2:0] out;

encod_8X3 uut (

.a(a),

.out(out)

);

initial begin

a = 8'b00000001; #10;

a = 8'b00000010; #10;

a = 8'b00000100; #10;

a = 8'b00001000; #10;

a = 8'b00010000; #10;

a = 8'b00100000; #10;

a = 8'b01000000; #10;

a = 8'b10000000; #10;

a = 8'b00000000; #10;

a = 8'b11111111; #10;

$finish;

end

endmodule

- Priority Encoder 8X3

module pencod_8X3(

input [7:0] a,

output reg [2:0] out);

always @(*) begin

casez (a)

8'b00000000 : out = 3'b000;

8'b00000001 : out = 3'b000;

8'b0000001? : out = 3'b001;

8'b000001?? : out = 3'b010;

8'b00001??? : out = 3'b011;

8'b0001???? : out = 3'b100;

8'b001????? : out = 3'b101;

8'b01?????? : out = 3'b110;

8'b1??????? : out = 3'b111;

endcase

end

endmodule

TB

module pencod_8X3_tb;

reg [7:0] a;

wire [2:0] out;

pencod_8X3 uut(a, out);

initial begin

a = 8'b00000000; #10;

a = 8'b00000001; #10;

a = 8'b00000010; #10;

a = 8'b00000100; #10;

a = 8'b00001000; #10;

a = 8'b00010000; #10;

a = 8'b00100000; #10;

a = 8'b01000000; #10;

a = 8'b10000000; #10;

a = 8'b11111111; #10;

a = 8'b00000011; #10;

a = 8'b00000111; #10;

a = 8'b00001111; #10;

$finish;

end

endmodule

- 1 Bit Comparator

module comp1(

input a, b,

output reg l ,e, g);

initial begin

e = 0;

l = 0;

g = 0;

end

always @(*) begin

if (a > b) begin

g = 1'b1; l = 0; e = 0;

end

else if ( a < b) begin

l = 1; g =0; e = 0;

end

else

e = 1; g = 0; l = 0;

end

endmodule

TB

module comp1_tb;

reg a, b;

wire l, e, g;

comp1 uut(a, b, l, e, g);

initial begin

a = 0; b = 0; #10;

a = 0; b = 1; #10;

a = 1; b = 0; #10;

a = 1; b = 1; #10;

$finish;

end

endmodule

- 2 Bit Comparator

module comp2(

input [1:0] a,b,

output reg l,e,g );

always @(*) begin

if (a>b) begin

g = 1;

l = 0;

e = 0;

end

else if (a<b) begin

l = 1;

g = 0;

e = 0;

end

else begin

e = 1;

l = 0;

g = 0;

end

end

endmodule

TB

module comp2_tb;

reg [1:0] a, b;

wire l, e, g;

comp2 uut(a, b, l, e, g);

initial begin

a = 2'b00; b = 2'b00; #10;

a = 2'b00; b = 2'b01; #10;

a = 2'b01; b = 2'b00; #10;

a = 2'b10; b = 2'b10; #10;

a = 2'b11; b = 2'b10; #10;

a = 2'b10; b = 2'b11; #10;

$finish;

end

endmodule

- 4 Bit Comparator

module comp4(

input [3:0] a,b,

output reg l,e,g );

always @(*) begin

if (a>b) begin

g = 1;

l = 0;

e = 0;

end

else if (a<b) begin

l = 1;

g = 0;

e = 0;

end

else begin

e = 1;

l = 0;

g = 0;

end

end

endmodule

TB

module comp4_tb;

reg [3:0] a, b;

wire l, e, g;

comp4 uut(a, b, l, e, g);

initial begin

a = 4'b0000; b = 4'b0000; #10;

a = 4'b0010; b = 4'b0001; #10;

a = 4'b0001; b = 4'b0010; #10;

a = 4'b1111; b = 4'b1111; #10;

a = 4'b1001; b = 4'b0110; #10;

a = 4'b0011; b = 4'b1010; #10;

$finish;

end

endmodule

- 8 Bit Comparator

module comp8(

input [7:0] a,b,

output reg l,e,g );

always @(*) begin

if (a>b) begin

g = 1;

l = 0;

e = 0;

end

else if (a<b) begin

l = 1;

g = 0;

e = 0;

end

else begin

e = 1;

l = 0;

g = 0;

end

end

endmodule

TB

module comp8_tb;

reg [7:0] a, b;

wire l, e, g;

comp8 uut(a, b, l, e, g);

initial begin

a = 8'd0;   b = 8'd0;   #10;

a = 8'd45;  b = 8'd32;  #10;

a = 8'd10;  b = 8'd75;  #10;

a = 8'd255; b = 8'd255; #10;

a = 8'd100; b = 8'd99;  #10;

a = 8'd5;   b = 8'd120; #10;

$finish;

end

endmodule

- Greater Than Comparator

module compgt(

input [7:0] a,b,

output reg g );

always @(*) begin

if (a>b)

g = 1;

else

g = 0;

end

endmodule

TB

module compgt_tb;

reg [7:0] a, b;

wire l, e, g;

compgt uut(a, b, l, e, g);

initial begin

a = 8'd0;   b = 8'd0;   #10;

a = 8'd45;  b = 8'd32;  #10;

a = 8'd10;  b = 8'd75;  #10;

a = 8'd255; b = 8'd255; #10;

a = 8'd100; b = 8'd99;  #10;

a = 8'd5;   b = 8'd120; #10;

$finish;

end

endmodule

- 4 Bit Carry Look Ahead Adder

module cla4 (

input [3:0] a, b,

input cin,

output [3:0] sum,

output cout

);

wire [3:0] g, p, c;

assign g = a & b;

assign p = a ^ b;

assign c[0] = cin;

assign c[1] = g[0] | (p[0] & c[0]);

assign c[2] = g[1] | (p[1] & g[0]) | (p[1] & p[0] & c[0]);

assign c[3] = g[2] | (p[2] & g[1]) | (p[2] & p[1] & g[0]) | (p[2] & p[1] & p[0] & c[0]);

assign sum = p ^ c;

assign cout = g[3] | (p[3] & g[2]) | (p[3] & p[2] & g[1]) |

(p[3] & p[2] & p[1] & g[0]) |

(p[3] & p[2] & p[1] & p[0] & cin);

Endmodule

TB

module cla4_TB;

reg [3:0] a, b;

reg cin;

wire [3:0] sum;

wire cout;

cla4 uut (

.a(a), .b(b), .cin(cin),

.sum(sum), .cout(cout)

);

initial begin

a = 4'b0000; b = 4'b0000; cin = 0;

#10 a = 4'b0001; b = 4'b0001; cin = 0;

#10 a = 4'b0011; b = 4'b0101; cin = 1;

#10 a = 4'b1111; b = 4'b0001; cin = 0;

#10 a = 4'b1010; b = 4'b0101; cin = 1;

#10 $finish;

end

endmodule

- 4 Bit Binary Multiplier

module mult4 (

input [3:0] a, b,

output [7:0] product

);

wire [7:0] p0 = b[0] ? {4'b0000, a}       : 8'b0;

wire [7:0] p1 = b[1] ? {3'b000, a, 1'b0}  : 8'b0;

wire [7:0] p2 = b[2] ? {2'b00, a, 2'b00}  : 8'b0;

wire [7:0] p3 = b[3] ? {1'b0, a, 3'b000}  : 8'b0;

assign product = p0 + p1 + p2 + p3;

endmodule

TB

module mult4_TB;

reg [3:0] a, b;

wire [7:0] product;

mult4 uut (

.a(a), .b(b),

.product(product)

);

initial begin

a = 4'b0000; b = 4'b0000;

#10 a = 4'b0010; b = 4'b0011;

#10 a = 4'b1111; b = 4'b0001;

#10 a = 4'b1010; b = 4'b0101;

#10 a = 4'b1111; b = 4'b1111;

#10 $finish;

end

endmodule

- Booth Multiplier

module booth_multiplier (

input  [3:0] multiplicand,

input  [3:0] multiplier,

output [7:0] product

);

reg [7:0] A, S, P;

reg [3:0] M, Q;

integer i;

always @(*) begin

A = {multiplicand, 4'b0000};

S = {~multiplicand + 1'b1, 4'b0000};

P = {4'b0000, multiplier, 1'b0};

for (i = 0; i < 4; i = i + 1) begin

case (P[1:0])

2'b01: P = P + A;

2'b10: P = P + S;

default: P = P;

endcase

P = {P[7], P[7:1]}; // arithmetic right shift

end

end

assign product = P[7:0];

endmodule

TB

module booth_multiplier_TB;

reg  [3:0] multiplicand, multiplier;

wire [7:0] product;

booth_multiplier uut (

.multiplicand(multiplicand),

.multiplier(multiplier),

.product(product)

);

initial begin

multiplicand = 4'd3;  multiplier = 4'd2;  #10;

multiplicand = 4'd4;  multiplier = 4'd3;  #10;

multiplicand = 4'd7;  multiplier = 4'd5;  #10;

multiplicand = 4'd8;  multiplier = 4'd8;  #10;

multiplicand = 4'd15; multiplier = 4'd15; #10;

$finish;

end

endmodule

- Array Multiplier

module array_multiplier (

input  [3:0] a,

input  [3:0] b,

output [7:0] product

);

wire [3:0] p0, p1, p2, p3;

wire [7:0] s1, s2, s3;

assign p0 = a & {4{b[0]}};

assign p1 = a & {4{b[1]}};

assign p2 = a & {4{b[2]}};

assign p3 = a & {4{b[3]}};

assign s1 = {1'b0, p1, 1'b0}; // shifted left by 1

assign s2 = {2'b0, p2, 2'b0}; // shifted left by 2

assign s3 = {3'b0, p3, 3'b0}; // shifted left by 3

assign product = {4'b0000, p0} + s1 + s2 + s3;

endmodule

TB

module array_multiplier_TB;

reg  [3:0] a, b;

wire [7:0] product;

array_multiplier uut (

.a(a),

.b(b),

.product(product)

);

initial begin

a = 4'd3;  b = 4'd2;  #10;

a = 4'd4;  b = 4'd5;  #10;

a = 4'd7;  b = 4'd3;  #10;

a = 4'd8;  b = 4'd8;  #10;

a = 4'd15; b = 4'd15; #10;

$finish;

end

endmodule

- Divider using Repeated Subtraction

module divider_subtract (

input      [3:0] dividend,

input      [3:0] divisor,

output reg [3:0] quotient,

output reg [3:0] remainder

);

integer i;

reg [3:0] temp_dividend;

always @(*) begin

quotient = 0;

remainder = 0;

temp_dividend = dividend;

if (divisor == 0) begin

quotient = 4'b0000;

remainder = dividend;

end else begin

while (temp_dividend >= divisor) begin

temp_dividend = temp_dividend - divisor;

quotient = quotient + 1;

end

remainder = temp_dividend;

end

end

endmodule

TB

module divider_subtract_TB;

reg  [3:0] dividend, divisor;

wire [3:0] quotient, remainder;

divider_subtract uut (

.dividend(dividend),

.divisor(divisor),

.quotient(quotient),

.remainder(remainder)

);

initial begin

dividend = 4'd10; divisor = 4'd3; #10;

dividend = 4'd15; divisor = 4'd4; #10;

dividend = 4'd9;  divisor = 4'd2; #10;

dividend = 4'd8;  divisor = 4'd0; #10;

dividend = 4'd7;  divisor = 4'd7; #10;

$finish;

end

endmodule

- D -Flip Flop

module d_ff (

input d,

input clk,

output reg q

);

always @(posedge clk)

q <= d;

endmodule

TB

module d_ff_TB;

reg d, clk;

wire q;

d_ff uut (

.d(d),

.clk(clk),

.q(q)

);

initial begin

clk = 0;

forever #5 clk = ~clk;

end

initial begin

d = 0; #10;

d = 1; #10;

d = 0; #10;

d = 1; #10;

d = 1; #10;

d = 0; #10;

$finish;

end

endmodule

- T Flip Flop

module t_ff (

input t,

input clk,

output reg q

);

always @(posedge clk) begin

if (t)

q <= ~q;

end

endmodule

TB

module t_ff_TB;

reg t, clk;

wire q;

t_ff uut (

.t(t),

.clk(clk),

.q(q)

);

initial begin

clk = 0;

forever #5 clk = ~clk;

end

initial begin

t = 0; #10;

t = 1; #10;

t = 1; #10;

t = 0; #10;

t = 1; #10;

t = 1; #10;

$finish;

end

endmodule

- JK Flip Flop

module jk_ff (

input j, k, clk,

output reg q

);

always @(posedge clk) begin

case ({j, k})

2'b00: q <= q;

2'b01: q <= 0;

2'b10: q <= 1;

2'b11: q <= ~q;

endcase

end

endmodule

TB

module jk_ff_TB;

reg j, k, clk;

wire q;

jk_ff uut (

.j(j),

.k(k),

.clk(clk),

.q(q)

);

initial begin

clk = 0;

forever #5 clk = ~clk;

end

initial begin

j = 0; k = 0; #10;

j = 0; k = 1; #10;

j = 1; k = 0; #10;

j = 1; k = 1; #10;

j = 0; k = 1; #10;

j = 1; k = 1; #10;

$finish;

end

endmodule

- SR Flip Flop

module sr_ff (

input s, r, clk,

output reg q

);

always @(posedge clk) begin

case ({s, r})

2'b00: q <= q;

2'b01: q <= 0;

2'b10: q <= 1;

2'b11: q <= 1'bx; // Invalid condition

endcase

end

endmodule

TB

module sr_ff_TB;

reg s, r, clk;

wire q;

sr_ff uut (

.s(s),

.r(r),

.clk(clk),

.q(q)

);

initial begin

clk = 0;

forever #5 clk = ~clk;

end

initial begin

s = 0; r = 0; #10;

s = 1; r = 0; #10;

s = 0; r = 1; #10;

s = 1; r = 1; #10;

s = 0; r = 0; #10;

$finish;

end

endmodule

- Master-Slave Flip-Flop

module ms_d_ff(

input d, clk,

output reg q

);

reg master;

always @(posedge clk)

master <= d;

always @(negedge clk)

q <= master;

endmodule

TB

module ms_d_ff_TB;

reg d, clk;

wire q;

ms_d_ff uut (

.d(d),

.clk(clk),

.q(q)

);

initial begin

clk = 0;

forever #5 clk = ~clk;

end

initial begin

d = 0; #10;

d = 1; #10;

d = 0; #10;

d = 1; #10;

d = 1; #10;

d = 0; #10;

$finish;

end

endmodule

- 4-bit Register with Positive Clock Edge

module reg4bit(

input [3:0] d,

input clk,

output reg [3:0] q

);

always @(posedge clk)

q <= d;

endmodule

TB

module reg4bit_TB;

reg [3:0] d;

reg clk;

wire [3:0] q;

reg4bit uut (

.d(d),

.clk(clk),

.q(q)

);

initial begin

clk = 0;

forever #5 clk = ~clk;

end

initial begin

d = 4'b0000; #10;

d = 4'b1010; #10;

d = 4'b1111; #10;

d = 4'b0101; #10;

d = 4'b0011; #10;

$finish;

end

endmodule

- 4-bit Shift Register (Left)

module shift_register_left(

input clk,

input rst,

input in,

output reg [3:0] q

);

always @(posedge clk or posedge rst) begin

if (rst)

q <= 4'b0000;

else

q <= {q[2:0], in};

end

endmodule

TB

module shift_register_left_TB;

reg clk;

reg rst;

reg in;

wire [3:0] q;

shift_register_left uut (

.clk(clk),

.rst(rst),

.in(in),

.q(q)

);

initial begin

clk = 0;

forever #5 clk = ~clk;

end

initial begin

rst = 1; in = 0; #10;

rst = 0; in = 1; #10;

in = 0; #10;

in = 1; #10;

in = 1; #10;

in = 0; #10;

$finish;

end

endmodule

- 4-bit Shift Register (Right)

module shift_register_right(

input clk,

input rst,

input in,

output reg [3:0] q

);

always @(posedge clk or posedge rst) begin

if (rst)

q <= 4'b0000;

else

q <= {in, q[3:1]};

end

endmodule

TB

module shift_register_right_TB;

reg clk;

reg rst;

reg in;

wire [3:0] q;

shift_register_right uut (

.clk(clk),

.rst(rst),

.in(in),

.q(q)

);

initial begin

clk = 0;

forever #5 clk = ~clk;

end

initial begin

rst = 1; in = 0; #10;

rst = 0; in = 1; #10;

in = 0; #10;

in = 1; #10;

in = 1; #10;

in = 0; #10;

$finish;

end

endmodule

- Bidirectional Shift Register

module shift_register_bidir(

input clk,

input rst,

input dir,         // 0 = left, 1 = right

input in,

output reg [3:0] q

);

always @(posedge clk or posedge rst) begin

if (rst)

q <= 4'b0000;

else if (dir)      // Right shift

q <= {in, q[3:1]};

else               // Left shift

q <= {q[2:0], in};

end

endmodule

TB
module shift_register_bidir_TB;

reg clk;

reg rst;

reg dir;

reg in;

wire [3:0] q;

shift_register_bidir uut (

.clk(clk),

.rst(rst),

.dir(dir),

.in(in),

.q(q)

);

initial begin

clk = 0;

forever #5 clk = ~clk;

end

initial begin

rst = 1; dir = 0; in = 0; #10;

rst = 0;

in = 1; #10; // left shift in 1

in = 0; #10; // left shift in 0

dir = 1;

in = 1; #10; // right shift in 1

in = 0; #10; // right shift in 0

$finish;

end

endmodule

- Parallel-In Parallel-Out Register

module pipo_register (

input clk,

input rst,

input [3:0] d,

output reg [3:0] q

);

always @(posedge clk or posedge rst) begin

if (rst)

q <= 4'b0000;

else

q <= d;

end

endmodule

TB

module pipo_register_TB;

reg clk;

reg rst;

reg [3:0] d;

wire [3:0] q;

pipo_register uut (

.clk(clk),

.rst(rst),

.d(d),

.q(q)

);

initial begin

clk = 0;

forever #5 clk = ~clk;

end

initial begin

rst = 1; d = 4'b0000; #10;

rst = 0; d = 4'b1010; #10;

d = 4'b1100; #10;

d = 4'b0111; #10;

$finish;

end

endmodule

- 4-bit Asynchronous Counter

module async_counter_4bit (

input clk,

input rst,

output reg [3:0] count

);

always @(negedge clk or posedge rst) begin

if (rst)

count <= 4'b0000;

else

count <= count + 1;

end

endmodule

TB

module tb_async_counter_4bit;

reg clk, rst;

wire [3:0] count;

async_counter_4bit uut (

.clk(clk),

.rst(rst),

.count(count)

);

initial begin

clk = 0;

forever #5 clk = ~clk;

end

initial begin

rst = 1;

#10 rst = 0;

#100 $finish;

end

endmodule

- 4-bit Synchronous Counter

module sync_counter_4bit(

input clk,

input rst,

output reg [3:0] count

);

always @(posedge clk or posedge rst) begin

if (rst)

count <= 4'b0000;

else

count <= count + 1;

end

endmodule

TB

module sync_counter_4bitTB;

reg clk, rst;

wire [3:0] count;

sync_counter_4bit uut (

.clk(clk),

.rst(rst),

.count(count)

);

initial begin

clk = 0;

forever #5 clk = ~clk;

end

initial begin

rst = 1;

#10 rst = 0;

#100 $finish;

end

endmodule

- Up Counter

module up_counter(

input clk,

input rst,

output reg [3:0] count

);

always @(posedge clk or posedge rst) begin

if (rst)

count <= 4'b0000;

else

count <= count + 1;

end

endmodule

TB

module up_counterTB;

reg clk, rst;

wire [3:0] count;

up_counter uut (

.clk(clk),

.rst(rst),

.count(count)

);

initial begin

clk = 0;

forever #5 clk = ~clk;

end

initial begin

rst = 1;

#10 rst = 0;

#100 $finish;

end

endmodule

- Down Counter

module down_counter(

input clk,

input rst,

output reg [3:0] count

);

always @(posedge clk or posedge rst) begin

if (rst)

count <= 4'b1111;

else

count <= count - 1;

end

endmodule

TB

module down_counterTB;

reg clk, rst;

wire [3:0] count;

down_counter uut (

.clk(clk),

.rst(rst),

.count(count)

);

initial begin

clk = 0;

forever #5 clk = ~clk;

end

initial begin

rst = 1;

#10 rst = 0;

#100 $finish;

end

endmodule

- Up/Down Counter

module up_down_counter(

input clk,

input rst,

input up_down, // 1: up, 0: down

output reg [3:0] count

);

always @(posedge clk or posedge rst) begin

if (rst)

count <= 4'b0000;

else if (up_down)

count <= count + 1;

else

count <= count - 1;

end

endmodule

TB

module up_down_counterTB;

reg clk, rst, up_down;

wire [3:0] count;

up_down_counter uut (

.clk(clk),

.rst(rst),

.up_down(up_down),

.count(count)

);

initial begin

clk = 0;

forever #5 clk = ~clk;

end

initial begin

rst = 1; up_down = 1;

#10 rst = 0;

#50 up_down = 0;

#50 $finish;

end

endmodule